Experiment 14

Simulating & Troubleshooting Network Failures

Lab 14 Objectives:

● Simulate and troubleshoot network failures due to misconfigured IPs or cable issues.

Prerequisites: The prerequisites of this lab are-

● Basic knowledge of networking components (IP addressing, subnetting, physical connections).

● Familiarity with troubleshooting tools and methodologies.

Outcomes: By the end of this lab, students will be able to:

● Identify and resolve IP misconfiguration issues.

● Detecting and troubleshooting network failures caused by physical layer problems.

● Use CLI tools to diagnose and restore network connectivity.

1. Simulate IP Misconfiguration:

Step: Assign an incorrect IP address manually to a system (for example, set it outside the subnet or same IP on two systems).

Expected Outcome: The device will fail to communicate with the network (no internet or local access).

Command to verify:

ipconfig (Windows)

ifconfig (Linux/macOS)

ping [gateway IP]

Common Issues Identified:

IP conflict (same IP on two devices)

Wrong subnet mask

Missing or incorrect default gateway

2. Simulate Physical Layer Failure:

Step: Disconnect the Ethernet cable or use a damaged/faulty cable.

Expected Outcome: The system will lose connection to the network.

Verification Steps:

- Check network icon (shows no internet or disconnected)

- Use ipconfig or ifconfig to see if the interface has an IP.

- Try to ping the default gateway – it will fail.

- Use a cable tester or replace the cable.

3. Troubleshooting using CLI Tools

Using ipconfig / ifconfig:

- View current IP address, subnet, and default gateway.

- Detect if the IP is in the correct range or is assigned as 169.254.x.x (automatic private IP – indicates DHCP failure).

Using ping:

- Test connectivity to:

  - Local gateway: ping 192.168.1.1

  - External host: ping google.com

Using tracert / traceroute:

- Trace the path to a host to see where the connection fails.

4. Fixing the Issues:

IP Configuration Fixes:

- Assign correct IP, subnet mask, and default gateway manually or enable DHCP.

- Example (Windows):

ipconfig /release

ipconfig /renew

Cable Fixes:

- Reconnect or replace Ethernet cables.

- Check physical ports and switch/router connectivity.

Documented CLI Outputs (Examples):

Incorrect IP (before fix):

After IP Fix:

Ping Result:

Summary Report:

Issues Simulated:

- Incorrect IP configuration (wrong subnet, duplicate IP).

- Cable disconnected or faulty Ethernet cable.

Troubleshooting Tools Used:

- ping

- ipconfig / ifconfig

- tracert / traceroute

Problems Identified:

- Device received APIPA (169.x.x.x) address due to DHCP failure.

- Physical layer failure due to unplugged Ethernet cable.

- Network unreachable via ping and traceroute.

Fixes Applied:

- Corrected IP settings using DHCP/manual configuration.

- Replaced damaged Ethernet cable with a working one.

Final Outcome:

All devices were successfully reconnected to the network after identifying and resolving both logical (IP misconfiguration) and physical (cable) issues.