Experiment 4

Simulating a Network with Different Devices

Lab 4 Objectives:

● Simulate a network with multiple devices using network simulation software

Prerequisites: The prerequisites of this lab are

- Basic understanding of networking concepts (IP addressing, subnets, routing).

- Familiarity with network simulation tools such as Cisco Packet Tracer, GNS3, or NetSim.

- Knowledge of networking protocols (TCP/IP, ICMP, HTTP, etc.)

Outcomes: By the end of this lab, students will be able to:

- Design and configure a network topology with different types of devices.

- Assign and verify IP configurations in a simulated network environment.

- Monitor and analyze network traffic to understand data flow.

Part 1: Setting Up the Network Simulation

Step 1: Selecting Network Devices

Open Cisco Packet Tracer.

Drag and drop the following devices:

- 1 Router (Router-PT)

- 1 Switch (2960 Switch)

- 2 PCs (PC0, PC1)

- 1 Server

Step 2: Establishing Network Connections

Use Copper Straight-Through cables to connect:

- PC0 to Switch (FastEthernet0/2)

- PC1 to Switch (FastEthernet0/3)

- Server to Switch (FastEthernet0/4)

Use Copper Straight-Through cable to connect Switch to Router:

Switch FastEthernet0/1 to Router GigabitEthernet0/0

Part 2: Configuring IP Addresses

Step 1: Assigning IPs to End Devices

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC0 | 192.168.1.10 | 255.255.255.0 |
| PC1 | 192.168.1.11 | 255.255.255.0 |
| Server | 192.168.1.20 | 255.255.255.0 |

Step 2: Configuring Router Interfaces

- Access the router CLI and enter the following commands:

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# exit

Router# write memory

Part 3: Verifying Network Configuration

Step 1: Testing Network Connectivity

- From PC0, open the command prompt and ping PC1:

From PC1, ping the Server:

Part 4: Analyzing Data Flow

Step 1: Capturing Network Traffic

To capture network traffic, we use tools like Wireshark or the simulation mode of Cisco Packet Tracer.

- Wireshark is a widely used packet analyzer that captures and analyzes real network traffic.

- Cisco Packet Tracer's simulation mode allows us to view packet flow within a virtual network.

Process of capturing packets:

- Initiate data transfer between network devices (for example, by using the ping command).

- Packets are captured by Wireshark or the Packet Tracer simulation tool.

- By applying filters, we can view only specific protocol packets, such as ICMP.

Filtering ICMP packets:
ICMP is the protocol used for ping requests and replies. By typing icmp in the filter bar, only ping-related packets will be displayed.

This process helps us analyze network traffic to understand data flow and verify network connectivity.

Step 2: Tracing Data Flow

- From PC0 command prompt, use traceroute to PC1: