Experiment 5

Connecting Devices to a Wi-Fi Network

Lab 5 Objectives:

● Connecting Devices to a Wi-Fi Network, Implementing WPA2 Security, and Testing Connectivity

Prerequisites: The prerequisites of this lab are-

● Basic knowledge of networking concepts and wireless communication.

● Familiarity with Wi-Fi security protocols and encryption standards.

● Access to a router with WPA2 security and wireless-enabled devices.

Outcomes: By the end of this lab, students will be able to:

● Configure a Wi-Fi network and connect different devices securely.

● Implement WPA2 encryption for wireless security.

● Test network connectivity and troubleshoot common Wi-Fi issues.

Part 1: Understanding Wi-Fi Networks and Security

Step 1: Wi-Fi Fundamentals

Overview of Wi-Fi Standards (802.11a/b/g/n/ac/ax):

Wi-Fi has various standards that define speed, range, and frequency:

- 802.11a: Uses 5 GHz band, speeds up to 54 Mbps, less interference.

- 802.11b: Uses 2.4 GHz band, speeds up to 11 Mbps, more interference.

- 802.11g: Uses 2.4 GHz band, speeds up to 54 Mbps, backward compatible with 802.11b.

- 802.11n: Dual band (2.4 & 5 GHz), speeds up to 600 Mbps, uses MIMO technology.

- 802.11ac: Uses 5 GHz band, speeds up to several Gbps, faster and better range.

- 802.11ax (Wi-Fi 6): Latest standard with improved efficiency, speed, and capacity, supports both 2.4 & 5 GHz.

Understanding SSID, Channels, Frequency Bands (2.4 GHz vs 5 GHz):

- SSID (Service Set Identifier): The unique name of a Wi-Fi network.

- Channels: Wi-Fi operates on specific channels within frequency bands. The 2.4 GHz band has 1-11 channels, often overlapping; 5 GHz has more non-overlapping channels.

- Frequency Bands:

  - 2.4 GHz: Longer range but slower speeds and more interference.

  - 5 GHz: Faster speeds, less interference, but shorter range.

Step 2: Wi-Fi Security Basics

Explanation of WEP, WPA, WPA2, and WPA3 Security Protocols:

- WEP (Wired Equivalent Privacy): An outdated and insecure protocol.

- WPA (Wi-Fi Protected Access): Improved security over WEP but still vulnerable.

- WPA2: Currently the standard security protocol using AES encryption, highly secure.

- WPA3: The newest security standard with enhanced protection against attacks.

Importance of Encryption:

Encryption protects wireless data from unauthorized access and ensures privacy by encoding the data transmitted over the network.

Part 2: Configuring and Securing a Wi-Fi Network

Step 1: Setting Up a Wi-Fi Network

- Access the router’s admin panel via a web browser by typing the router’s IP address (commonly 192.168.1.1 or 192.168.0.1).

- Login with the admin username and password.

- Set the SSID (network name).

- Select the network mode (such as 802.11n/ac/ax).

- Set a secure password for the Wi-Fi network.

Step 2: Implementing WPA2 Security

- Enable WPA2-Personal security mode in router settings.

- Choose AES as the encryption method for strong security.

- Generate and set a strong passphrase containing uppercase, lowercase letters, numbers, and special characters.

- Save the settings.

Example:   123456780           Akdfudfni*@7990

Step 3: Connecting Devices to the Network

- On the laptop, smartphone, and IoT devices, open Wi-Fi settings.

- Select the SSID configured earlier.

- Enter the WPA2 password and connect.

- Verify devices are connected using WPA2 encryption.

Part 3: Testing and Analyzing Network Connectivity

Step 1: Checking Connectivity

- Use ping command in Command Prompt or Terminal to test internet connectivity, e.g.

- Use tracert (Windows) or traceroute (Linux/macOS) to view the path data takes to reach a website:

- Check both internet and local network connectivity.