Experiment 6

ARP & PPP Configuration

Lab 6 Objectives:

● Observe ARP message exchange and configure PPP with authentication for direct network connections.

Prerequisites: The prerequisites of this lab are-

● Basic networking knowledge (MAC & IP addressing, ARP, PPP).

● Familiarity with command-line tools for network configuration.

Outcomes: By the end of this lab, students will be able to:

● Capture and analyze ARP messages.

● Configure and verify PPP with authentication.

● Troubleshoot ARP and PPP-related issues.

What is ARP (Address Resolution Protocol)?

ARP is used to map IP addresses to MAC (hardware) addresses in a local area network. It is essential for devices to communicate within the same subnet.

How ARP Works:

- If a device wants to send data to another device but only has its IP address, it sends an ARP Request (broadcast).

- The target device replies with an ARP Reply (unicast), providing its MAC address.

- The sender stores the MAC-IP mapping in its ARP cache.

What is PPP (Point-to-Point Protocol)?

PPP is a data link layer protocol used to establish a direct connection between two networking nodes (usually routers) over serial links.

Features of PPP:

- Supports authentication using PAP or CHAP.

- Encapsulates network layer protocols.

- Error detection and compression.

PPP Authentication Methods:

1. PAP (Password Authentication Protocol):

- Two-way handshake.

- Transmits username and password in plain text.

- Less secure.

2. CHAP (Challenge Handshake Authentication Protocol):

- Three-way handshake.

- Encrypts the password using a hash function.

- More secure.

Step 2: Capture ARP Packets (Practical Demonstration)

On PC1 (my laptop), I used the ping command to initiate communication with another device in the same subnet:

This confirms that the destination device is reachable, and the system successfully resolved the IP address using ARP.

Then, using Wireshark, I captured the ARP message exchange. I applied the filter:

Observation:

ARP Request: Who has 192.168.226.42? Tell 192.168.226.177

ARP Reply: 192.168.226.42 is at dc:65:56:a0:19:93

Configure PPP Between Two Routers – Theory

Objective:

To configure a Point-to-Point Protocol (PPP) connection between two routers using serial interfaces for direct communication.

What is PPP?

Point-to-Point Protocol (PPP) is a Data Link Layer (Layer 2) protocol that is used to establish a direct connection between two networking devices such as routers over serial links. PPP supports:

- Encapsulation of network layer protocols (like IP, IPX)

- Authentication mechanisms (PAP and CHAP)

- Compression and error detection

Why Use PPP?

Although routers by default use HDLC (High-Level Data Link Control) as their encapsulation protocol on serial interfaces, PPP is more versatile and supports

- Multivendor compatibility

- Advanced authentication (PAP/CHAP)

- Link quality monitoring

Therefore, PPP is widely used in WAN (Wide Area Network) links where authentication and reliability are important.

Required Setup:

To configure PPP, we need:

- Two routers with serial interfaces

- A serial (DCE/DTE) cable to connect the routers

- IP addresses on both interfaces that belong to the same subnet

Configuration Commands:

On Router1:

On Router2:

Explanation:

- interface Serial0/0/0: Enters the serial interface configuration mode.

- ip address: Assigns an IP address in the same subnet to the interface.

- encapsulation ppp: Changes the default encapsulation from HDLC to PPP.

- no shutdown: Activates the interface.

Expected Outcome:

After configuring PPP on both routers:

- The serial link becomes active.

- PPP is used for encapsulation.

- The routers can communicate with each other using IP over a PPP link.

- This setup can later be extended to add authentication (CHAP/PAP).

Analysis Report

Experiment Title:

Observing ARP & Configuring PPP Authentication

The primary objective of this experiment was to:

- Understand the working of Address Resolution Protocol (ARP) in mapping IP addresses to MAC addresses within a local network.

- Configure and test Point-to-Point Protocol (PPP) with authentication methods like CHAP to secure communication over serial links.

Findings:

- ARP Functionality:
The experiment demonstrated that ARP plays a vital role in allowing devices to communicate over a LAN by translating IP addresses into MAC addresses. This translation is essential for packet delivery within the same network.

- PPP with CHAP Authentication:
The PPP configuration using CHAP (Challenge Handshake Authentication Protocol) ensured a more secure authentication process between two routers. The CHAP method involves a three-way handshake and periodically verifies the identity of the peer.

Problems Faced:

- Authentication Failure:
Initially, PPP authentication using CHAP failed because the username and password were mismatched between the two routers.

- Missing ARP Packets:
ARP requests and replies were not visible during packet capture when the network cable was disconnected, causing devices to fail in resolving MAC addresses.

Solutions Implemented:

- Correcting Authentication Configuration:
The username and password were rechecked and updated to ensure both routers had matching credentials for CHAP authentication.

- Enabling Interface:
The serial interface was reactivated using the **no shutdown** command, restoring connectivity and allowing ARP packets to be exchanged successfully.