Experiment 7

Subnetting & IP Assignment

Lab 7 Objectives:

● Perform subnetting calculations and assign appropriate IPs to networked systems.

Prerequisites: The prerequisites of this lab are-

● Basic understanding of IP addressing and subnetting.

● Familiarity with binary conversion and subnet mask calculations.

Outcomes: By the end of this lab, students will be able to:

● Calculate subnet masks and determine network and host ranges.

● Assign IP addresses to devices based on subnet requirements.

● Verify network connectivity and troubleshoot IP conflicts.

Step 1: Determine Subnet Requirements

Given network: 192.168.10.0/24
Requirement: Divide this network into 4 subnets with at least 50 hosts per subnet.

- Number of subnets required = 4

- Minimum hosts per subnet = 50

Step 2: Calculate the Subnet Mask

Default Information

- The IP address 192.168.10.0 is a Class C IP address.

- Default subnet mask for Class C is 255.255.255.0 or /24.

- This means 8 bits are available for host addresses (last octet).

Calculate number of bits to borrow for subnetting

To create 4 subnets:

2n ≥ 4 ⟹ n = 2 bits borrowed

- Borrow 2 bits from the host portion for subnetting.

- Remaining bits for hosts = 8 - 2 = 6 bits.

Calculate maximum hosts per subnet

26− 2 = 64 – 2 = 62 usable hosts per subnet

(2 addresses are reserved for network address and broadcast address)

Since 62 ≥ 50, this subnet mask satisfies the host requirement.

Calculate the new subnet mask

- Borrowing 2 bits means the new subnet mask is:

255.255.255.(256−28−2)=255.255.255.192

- In CIDR notation, this is /26.

Step 3: Find Subnet Addresses and Usable IP Ranges

Each subnet has 64 IP addresses (including network and broadcast addresses).

| Subnet | Network Address | Usable Host IP Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 192.168.10.0 | 192.168.10.1 – 192.168.10.62 | 192.168.10.63 |
| 2 | 192.168.10.64 | 192.168.10.65 – 192.168.10.126 | 192.168.10.127 |
| 3 | 192.168.10.128 | 192.168.10.129 – 192.168.10.190 | 192.168.10.191 |
| 4 | 192.168.10.192 | 192.168.10.193 – 192.168.10.254 | 192.168.10.255 |

Step 4: Assign IP Addresses to Devices

Assign unique IP addresses within the usable range to each device on a subnet. For example:

| Device | Subnet | Assigned IP | Subnet Mask |
| --- | --- | --- | --- |
| PC1 | 1 | 192.168.10.1 | 255.255.255.192 (/26) |
| PC2 | 1 | 192.168.10.2 | 255.255.255.192 (/26) |
| PC3 | 2 | 192.168.10.65 | 255.255.255.192 (/26) |
| PC4 | 3 | 192.168.10.130 | 255.255.255.192 (/26) |

Step 5: Configure and Verify IP Settings

- Configure IP addresses and subnet masks on devices.

- To check IP configuration on Windows, use the command:

ipconfig /all

Use tracert (Windows)

Step 6: Troubleshoot IP Conflicts and Connectivity Issues

If ping fails, check the following:

- IP address and subnet mask are configured correctly.

- No duplicate IP addresses on the network.

- Network cables or virtual links are connected properly.

- Devices are powered on and reachable.