Experiment 10

Subnetting and Creating Subnet Masks

Lab Objective:

To perform subnetting on a given IP address and create subnet masks to divide a network into smaller, efficient sub-networks.

What is Subnetting?

- Subnetting is the process of dividing a large IP network into smaller sub-networks (subnets).

- It helps in better IP address management, network performance, and security.

- Each subnet acts like a separate network but still remains part of the original large network.

Why Subnetting is Important

- Efficient utilization of IP addresses

- Reduces broadcast traffic

- Improves network performance and manageability

- Enhances security by isolating network sections

- Required in large organizations with multiple departments

Understanding IP Classes and Default Subnet Masks

| Class | Starting IP | Range | Default Subnet Mask |
| --- | --- | --- | --- |
| A | 1.0.0.0 | 1-126 | 255.0.0.0 (/8) |
| B | 128.0.0.0 | 128-191 | 255.255.0.0 (/16) |
| C | 192.0.0.0 | 192-223 | 255.255.255.0 (/24) |

Performing Subnetting (IPv4)

Suppose you are given:

- IP Address: 192.168.1.0

- Requirement: 4 subnets

Steps:

- Determine the subnet bits:

  - For 4 subnets, you need 2 bits (2² = 4)

- Modify subnet mask:

  - Default mask: /24 → add 2 bits → New mask: /26

  - Subnet Mask: 255.255.255.192

- Calculate subnet ranges:

| Subnet | Network Address | First Host | Last Host | Broadcast Address |
| --- | --- | --- | --- | --- |
| 1 | 192.168.1.0/26 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| 2 | 192.168.1.64/26 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| 3 | 192.168.1.128/26 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 |
| 4 | 192.168.1.192/26 | 192.168.1.193 | 192.168.1.254 | 192.168.1.255 |

Creating Subnet Masks (Binary to Decimal)

- /26 in binary:
11111111.11111111.11111111.11000000

- In decimal:
255.255.255.192

Subnetting in IPv6

- IPv6 already uses prefix length to divide networks (no subnet masks like IPv4)

- Example:

  - IPv6: 2001:db8:abcd::/48 → Want 4 subnets

  - Add 2 bits to /48 → New prefix: /50

  - Subnets:

    - 2001:db8:abcd:0::/50

    - 2001:db8:abcd:4000::/50

    - 2001:db8:abcd:8000::/50

    - 2001:db8:abcd:c000::/50

Verification Commands

On Windows (IPv4):

ipconfig

ping <other subnet host>

On Cisco Router:

show ip interface brief

To assign IP:

interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.192

no shutdown

Outcome:

After completing this experiment, students will be able to:

- Understand and perform subnetting

- Create custom subnet masks

- Divide a network logically using IPv4 and IPv6

- Improve network management and address allocation