Experiment 05

LAN Installation and Configuration

Objective:

To install and configure a Local Area Network (LAN) so that multiple computers can communicate and share resources within a limited area like a computer lab or classroom.

Prerequisites:

Before performing this experiment, students should have:

- Basic knowledge of computer networking.

- Understanding of network devices like switches, routers, cables, NIC.

- Awareness of IP addressing (IPv4).

What is LAN?

A Local Area Network (LAN) is a network that connects computers and devices in a limited geographical area, such as a room, building, or campus. It allows devices to share resources like files, printers, and internet connection.

Key Features of LAN:

- Covers small area (home, lab, office).

- High-speed communication (up to 1 Gbps).

- Allows file, printer, and internet sharing.

- Secure and easy to maintain.

Common LAN Devices:

| Device | Function |
| --- | --- |
| NIC (Network Interface Card) | Allows computer to connect to the LAN |
| Switch | Connects multiple devices in the LAN |
| Router | Connects LAN to the internet (if needed) |
| Ethernet Cable | Physical medium for connection (Cat5e/Cat6) |
| RJ45 Connector | Connector at the end of Ethernet cables |

Apparatus Required:

- 2 or more Computers / Laptops

- Network Switch or Router

- Ethernet (LAN) Cables – Cat5e/Cat6

- RJ45 connectors

- Crimping Tool (optional)

- Power Source

Procedure:

Step 1: Setup Devices

- Place computers close to a switch or router.

- Plug the power supply for all devices.

- Check if all computers have working LAN ports and NIC drivers installed.

Step 2: Make the Connections

- Connect each computer to the switch/router using Ethernet cables.

- Ensure all cables are tightly plugged into the RJ45 port.

- Power on all devices.

Step 3: Configure IP Addresses

Method 1: Static IP

- Go to: Control Panel → Network and Sharing Center

- Click: Change adapter settings → Right-click on Ethernet → Properties

- Select: Internet Protocol Version 4 (TCP/IPv4)

- Choose: "Use the following IP address"

  - IP Address: 192.168.1.X (X = 1,2,3…)

  - Subnet Mask: 255.255.255.0

  - Default Gateway: 192.168.1.1

🔸 Method 2: DHCP (Automatic IP)

- If using a router, enable DHCP.

- Computers will automatically receive IP addresses.

Step 4: Test LAN Connectivity

- Open Command Prompt on one computer.

- Type: ping 192.168.1.2 (another PC’s IP)

- If reply is received, LAN is working.

Reply from 192.168.1.2: bytes=32 time<1ms TTL=128

Step 5: File and Folder Sharing (Optional)

- Right-click any folder → Properties → Sharing → Share.

- Ensure network discovery and file sharing are enabled:

  - Go to: Control Panel → Network and Sharing Center → Advanced sharing settings.

Result:

Successfully configured a LAN network that enables communication between connected computers.

Conclusion:

From this experiment, we learned:

- How to set up a LAN using switch/router and Ethernet cables.

- How to assign IP addresses using static or DHCP methods.

- How to test network connectivity using the ping command.

- Basic resource sharing in a LAN.

Advantages of LAN:

| Advantage | Explanation |
| --- | --- |
| High Speed | Fast data transmission within local area |
| Resource Sharing | Devices can share printers, files, internet |
| Low Cost | Economical setup and maintenance |
| Secure | Easier to manage and protect from outside access |