Experiment 07

Configuration and Use of Network Devices

Lab Objective:

To configure and understand the functions of various network devices such as routers, switches, and hubs, and learn their role in a network setup.

Prerequisites:

- Basic understanding of networking concepts (e.g., IP addressing, data transmission).

- Familiarity with network configuration commands and tools (e.g., Cisco Packet Tracer, command line interface, IP configuration).

Outcome:

By the end of this experiment, students will be able to:

- Identify different network devices (router, switch, hub).

- Understand how they function in a network.

- Successfully configure routers and switches using commands/tools.

- Demonstrate data flow through a simple network.

1. Hub

- Definition: A hub is a basic networking device that connects multiple computers in a LAN.

- Function: It simply broadcasts data to all ports, regardless of the destination.

- Type: Physical Layer device (Layer 1 of OSI model).

- Working: When a data packet arrives at one port, it is copied to all other ports.

- Configuration:
No configuration needed — plug and play.

- Advantages:

  - Cheap and easy to use.

- Disadvantages:

  - No intelligence (no MAC filtering).

  - Causes network congestion.

- Use Case: Small or temporary networks for testing.

2. Switch

- Definition: A switch is a networking device that connects devices in a LAN and uses MAC addresses to forward data only to the intended device.

- Function: Makes decisions based on the destination MAC address.

- Layer: Data Link Layer (Layer 2).

- Working:

  - Maintains a MAC address table.

  - Learns the devices connected to each port and sends data to the correct port.

- Basic Configuration (using CLI):

- Advantages:

  - Reduces network traffic.

  - More secure than hubs.

- Disadvantages:

  - More expensive than hubs.

- Use Case: Office or campus LAN environments.

3. Router

- Definition: A router is a device that connects multiple networks and forwards data packets between them based on IP addresses.

- Function: Routes data between different networks (e.g., LAN to WAN).

- Layer: Network Layer (Layer 3).

- Working:

  - Uses routing tables and routing protocols (RIP, OSPF) to determine the best path.

- Basic Configuration (using CLI):

- Advantages:

  - Enables communication between different networks.

  - Provides firewall and security features.

- Disadvantages:

  - More complex and expensive.

- Use Case: Internet access, connecting LANs to WANs.

🔗 Comparison Table:

| Device | OSI Layer | Intelligent? | Forwards by | Configuration Needed | Use Case |
| --- | --- | --- | --- | --- | --- |
| Hub | Layer 1 | No | Broadcasts | No | Small testing networks |
| Switch | Layer 2 | Yes | MAC address | Basic VLAN setup | LAN communication |
| Router | Layer 3 | Yes | IP address | IP Routing setup | Internet / WAN connectivity |

Sample Network Setup:

Let’s assume a simple network:

- 2 PCs connected to a switch.

- Switch connected to a router.

- Router connects to another network or Internet.

Steps:

- Assign IP addresses to PCs (e.g., 192.168.1.2 and 192.168.1.3).

- Configure switch (optional VLAN).

- Configure router interfaces to connect internal and external networks.

- Test connectivity using ping command.

Conclusion:

Understanding and configuring network devices like hubs, switches, and routers is essential for building and maintaining a functional network. These devices ensure proper data delivery, efficiency, and security in communication.